Router is a high performance hard-RTOS network element/device with specialized network processing units (NPUs) for fast/real-time packet processing (tries/TCAM).

A Routing Algorithm is a mathematical procedure that a dynamic router uses to calculate entries for its routing table. It serves as the guiding mechanism that navigates data packets from their starting point to their intended endpoint.

A routing algorithm is essentially a set of rules or protocols designed to determine the most efficient path for data packets to travel across a network.

Think of it as a GPS for your data, calculating the best routes from the source to the destination, bypassing congestion and roadblocks.

Dynamic routing, is where routing algorithms come into play. These algorithms automatically update the routing table in real-time based on network conditions. Unlike its static counterpart, dynamic routing can adapt to changes, making it more flexible and reliable.

Dynamic routing algorithms are indispensable. They consider multiple factors such as bandwidth, network congestion, and even the reliability of a link to decide the most efficient path for data transmission. By continuously evaluating these parameters, routing algorithms ensure that the network adapts to any changes, providing optimal performance and reliability. They are the brain of routing, constantly recalibrating to ensure data packets follow the most efficient path.

A routing Algorithm is implemented as a routing protocol because it involves procedures and protocols that allow routers to exchange information with one another in order to calculate the metrics of various paths or routes through an internetwork. Routing algorithms base their work on the values contained in a combination of variables. These values can be determined dynamically by inspecting header information in packets directed toward the router, or they can be manually specified by network administrators. The routing algorithm then processes the values of these variables to generate the internal routing table for the router.

Some of the routing metrics used or gathered from packet headers,

- Hops, the number of intermediate routers between/from source and destination.

- Latency, the time delay in processing a packet through a router or over a given route.

- Congestion, the length of the packet queue at the incoming interface/port of a router.

- Load, processor use at the router – the number of packets it is processing per sec. (Datapath throughput)

- Bandwidth, the available capacity of a route to support network traffic, decreases as network traffic increases.

- Reliability, the relative amount of downtime that a particular router might experience because of malfunctions.

- Maximum Transmission Unit (MTU), The largest packet size that a router can forward without fragmentation.

Routing algorithms are often implemented as a combination of dynamic (real-time calculated) or static (entered by network administrators). The algorithms are distributed. With each router calculating its own routing tables independently, exchanging routing information with each other. If one router goes down all other routers can reconfigure their routing tables to route traffic around the failed router. When the failed router is restored the routing tables are recalculated. There is no single point of failure, making it robust and fault-tolerant. Several alternate paths to a destination are maintained to better manage network traffic i.e. Packet Load Balancing.

- From a mathematical point of view, distributed routing algorithms come in two common types: link state routing algorithms and distance vector routing algorithms. Routers using the distance vector algorithm periodically send their entire routing table to other routers, but only to routers one hop away, while the link state algorithm floods the entire internetwork with information from each router, but only updated information is sent. Link state algorithms require more processing power and are more costly but more precise/accurate, while distance vector routing algorithm is simpler to implement.

- Another routing concept is the routing space which can be either Flat or Hierarchical. In Flat space all routers are peers, while in Hierarchical space there are different Domains, Areas or Autonomous Systems connected using a backbone routing network.

- The link state algorithm is a hierarchical routing space one that forms the basis of the Open Shortest Path First (OSPF) protocol, while the distance vector algorithm is a flat routing space one that forms the basis of the Routing Information Protocol (RIP).

- The advantage of a hierarchical routing space is that it reduces the amount of intercommunication traffic that must take place between routers in order for them to calculate their routing tables. For example, routers that forward traffic only within their own routing table do not need to exchange routing information with routers in other domains.

- The downside, of course, is that a hierarchical system is much more difficult to implement and maintain than a flat routing space. Routing loops are less likely to occur when the link state algorithm is used.

Further Dynamic Adaptive Routing algorithms which change their routing decisions to adapt to network conditions, are divided into 3 types,

- Centralized: In this setup a single central node –usually a powerful router or server—calculates routes for all the routers in the network. It gathers all the network information, runs the routing algorithm, and then sends the updated routing tables to the other routers.

- Isolation: Routers operate independently without much cooperation from their neighbours to update its routing table. They make their routing decisions based on locally available information such as queue lengths at adjacent routers or current traffic conditions.

- Distributed: Link state and Distance vector algorithms

Non-Adaptive Routing Algorithms: They don’t change their routing decisions based on network conditions. They are simpler and quicker but not as flexible in adapting to network changes.

- Flooding: Every incoming packet is sent through every outgoing link except the one it arrived on. This is a simple yet resource-intensive method and is generally used in controlled environments or for specific use-cases like network discovery. E.g. ARP

- Random Walks: In this method, packets are moved randomly from one node to another until they reach their destination. While not efficient, this method is easy to implement and is sometimes used in scenarios where network topology is not well-defined. Random walks are often seen in mesh networks where there’s no clear path, and the packet eventually finds its way to the destination through random hops.

Algorithms to study in depth for their deployment properties in ISP/enterprise.

Open Shortest Path First (OSPF): Adaptive, Hierarchical, Link state. Supports multiple equal-cost routes, robust, and scalable. Large enterprise networks

Routing Information Protocol (RIP): Adaptive, Flat, Distance vector. Easy to configure but less efficient for larger networks. Small to medium-sized networks.

Border Gateway Protocol (BGP): Adaptive, Hierarchical Routing Space, path vector. Used for routing between autonomous systems, very scalable. Internet backbone routing.

More mathematical classification on which adaptive algorithms are based on,

### Dijkstra’s Algorithm: Adaptive, Hierarchical Routing Space, Link State. Finds the shortest path in a weighted graph, foundational for many other algorithms. OSPF uses a variant of Dijkstra’s algorithm. Link state are more complex but offer higher precision and are generally used in hierarchical routing spaces.

### Bellman-Ford Algorithm: Adaptive, Flat Routing Space, Distance Vector. Simple and effective but slower than Dijkstra’s for finding the shortest path. Used in RIP. Simpler to implement but generally less precise, these are often used in flat routing spaces.

### So, we’ve covered the basics of Routers. Most of the topics covered about routing algorithms and routing tables which is the fundamental of a router, address the control plane not the data plane. We now take up what IP addresses look like.

### Classless Addressing: Classless Inter-Domain Routing (CIDR) is a technique used to assign a block of IP addresses based on specified conditions when the user demands a specific amount of IP addresses. This block is known as a CIDR-block and it contains the necessary number of IP addresses. It follows these rules,

- The block IP addresses must all be contiguous.

- The block size must be a power of 2. Furthermore, the block’s size must be equal to the number of IP addresses in the block.

- The block’s first IP address must be divisible by block size.

### CIDR is a more efficient routing mechanism than the original method of segregating network IP addresses into classes named class A(1 byte), class B(2 byte), and class C(3 byte). It should now be clear that the constrains on classful addressing on byte boundary created the inefficient allocation of network/host IP addresses, where a few addresses worth IP would require a bump of a full class or a byte in addressing, where only 1 or 2 bits would have sufficed for required IP demand. It allows any number of contiguous bits in the IP address to be allocated as the network ID, like 27 bits and 5 bits for hosts.

CIDR assigns a numerical prefix to each IP address. For example, a typical destination IP address using CIDR might be 177.67.5.44/13. The prefix 13 indicates that the first 13 bits of the IP address identify the network, while the remaining 32 – 13 = 19 bits identify the host. The prefix helps to identify the Internet destination gateway or group of gateways to which the packet will be forwarded. Prefixes vary in size, with longer prefixes indicating more specific destinations.

Routers use the longest possible prefix in their routing tables when determining how to forward each packet. CIDR enables packets to be sent to groups of networks instead of to individual networks, which considerably simplifies the complex routing tables of the Internet’s backbone routers. The router at which a packet arrives is only the 32 bit IP address it does not have a netmask associated with it. Netmasks are maintained by a routers routing table, where routes containing network IDs are kept in longest prefix (network ID) order. Comparison is made between routing table entries and packet destination IP address matching bit-by-bit the network ID using the network prefixes on the packet. The route with the longest matching prefix is chosen for forwarding in the network using gateway associated with the network route.

### E.g. packet with destination IP 192.168.2.82 gets forwarded using routing entry 192.168.2.80/29. The bit pattern for 29 bits is an exact match with the packet and is the longest network prefix route in the routing table.

### TCAM (Ternary Content Addressable Memory)

### One of the key components of virtually any layer 2 or layer 3 switch is the Ternary Content Addressable Memory, or TCAM in short. TCAM is an expensive component and is typically a scarce resource on many switching platforms. Compared to traditional RAM as found in your laptop, CAM operates in a completely different way. Traditional RAM will simply return the content of memory cells at the address you specify when you want to access content stored in RAM. When working with CAM, you don’t know the specific memory location where the content you are interested in is located - but you know what you want to find. So you use a binary key, which the CAM uses to search its entire memory for occurrences of that specific key. For each occurrence found, the CAM will return the address of where to find that specific content. Typically, the content (or more precisely key) will be the MAC address and/or the VLAN ID of a connected end-host, and as such used to return relevant information pertaining to that specific MAC address, such as the associated VLAN and/or the port where it was last seen.

All searches are performed using an exact match on the key that was input to the search operation, so if we're looking for MAC address 000D.29A8.B28E, the search will only return memory addresses where the MAC address matches completely to the content in the memory, so the search won't return 000D.29A8.B28D or 000D.29A8.B38E. A CAM is often referred to as a binary CAM due to its ability to match only on 0's and 1's. The information returned from a binary search includes the VLAN and/or physical port, which allows the switch to forward the traffic to the correct egress port.

Switches uses an extension of a CAM, known as the TCAM or Ternary Content Addressable Memory. Whereas the binary CAM can match only on the traditional binary zeroes and ones, TCAM also can match a third state: any value or the don't care. This third state is called 'X', so a TCAM can work with three different values: '0', '1', and 'X', hence the name 'Ternary'. So, if the value '0111XX10' is associated with a TCAM entry, searching for either of the following values will result in a match:

01110010

01110110

01111010

01111110

The 'X' value is not stored as an 'X', but is implemented using masks, which in many aspects resembles the subnet masks/prefix lengths used in the IPv4 and IPv6 world. Mask values is what allows ACLs to be efficiently compiled and programmed to the TCAM. The process of compiling ACLs and programming them into the TCAM is commonly referred to as 'merging ACLs into TCAM' and is handled by the Feature Manager (FM), which is a software component inside the switch.

ach TCAM entry consist of three components: Value, Mask and Result. Both the value and mask fields are 134 bits in length, but it does not necessarily mean that all 134 bits will be actively used. The actual bit utilization depends on the type of ACL being implemented. Regardless of ACL types, value and mask fields use the exact same bit order. If a mask bit is set means that the corresponding bit position in the value field must match, while a mask bit that is not set means that the corresponding bit position in the value field does not matter. Results are implemented as numeric values that represent the action to take after the TCAM lookup. Beside the basic permit/deny actions found with traditional access lists, the result can also contain a pointer to a next-hop routing table or an index to a QoS policer etc.

TCAM entries are organized by the masks, with each unique mask having up to eight values associated with it. This allows each of the mask-value pairs to be evaluated simultaneously, so the best or longest match is found with a single lookup operation, which contributes to the fast operation of the TCAM.

### Border Gateway Protocol

### Internet is divided into many networks & routers owned by single authoritative authority and ISPs. Both put together define Autonomous Systems (AS). BGP is an exterior gateway protocol designed for information exchange about summary of reachability between interior routes to peer AS(s). Interior routing is either distance vector or link-state routing. The question is about the total network bandwidth utilization by routing protocols themselves as against normal traffic. It would be impractical to route to all other routers in the internet, the traffic from these protocols would be overwhelming and thus impractical. So, we have built groups around these routers and limit the size of participating routers to about a dozen in WAN and 5 times of that on the LAN. Automation in routers is not practiced by AS(s) in general, routes are most often added by network managers(policies) in the interest of revenues, low financial costs & competition. They are not the optimal shortest paths.